Surface Mount the supplemental Instruction

- Follow the standard instructions for the product assembly.

- Determine the length the post should be for the desired product height. Cut off the bottom portion of the post so that the leg is the required length. De-bur the end of the post and use the touch up paint provided to paint the cut end of the post.

- Insert surface mount into the bottom end of the post.

- Use a 3/16” drill to make a hole approximately 2" above the base plate.

- Insert sheet metal screw provided into the drilled hole to secure the post to the surface mount.

- The anchoring method is up to the installer we do not supply the anchors for attaching to the surface.